MOTOROLA KEYPRESS FORMAT

Keypress for some Motorola phones, including the p280, v60, v66, v120c. It doesn't work with some very recent v120t and v60 phones.

Entering Notes:

To create a new tone, use the keypad key to enter each note. Press a key multiple times to cycle through its available notes or options (pitch, octave, or length). The default setting for a new tone is a quarter note in octave two.

| Key | Phone Display | Description |
| --- | --- | --- |
| (1) | 1 | Set Octave One |
|  | 2 | Set Octave Two |
|  | 3 | Set Octave Three |
| (2) | A | Note ‘A’ |
|  | B | Note ‘B’ |
|  | C | Note ‘C’ |
| (3) | D | Note ‘D’ |
|  | E | Note ‘E’ |
|  | F | Note ‘F’ |
| (4) | G | Note ‘G’ |
|  | R | Rest |
| (7) | # | Sharp |
|  | b | Flat |
| (0) | q | Quarter note or Quarter rest |
|  | h | Half note or Half rest |
|  | w | Whole note or Whole rest |

Enter a Note as Follows:

| Change the octave | Set the new octave (1, 2, 3) before selecting the note. The new note applies to the note and all following notes until you change it again |
| --- | --- |
| Select the note (required) | Press a keypad key to enter a note |
| Change a note to sharp or flat | Enter a sharp or flat (# or b) after selecting the note. Some sharps and flats are played as standard notes. For example, B# is the same as C. |
| Change a note’s length | Select the new length (q, h, or w) after selecting the note. The new length applies to the note and all following notes until you change it again. |
| Add a rest | Enter one or more rests (R character) as needed in the tone sequence. You can set the length of the rest by entering a q, h, or w character after the rest, just as you can for a note. |

Example:

Press the following keys to play this sequence of notes and rests in octave three: C (quarter note), E flat (quarter note), half rest, and G (whole note):

| Press | # To | ## Phone Display |
| --- | --- | --- |
| (1) (1) (1) | Set octave three | 3 |
| (2) (2) (2) | Enter a quarter note C | C |
| (3) (3) (7) (7) | Enter a quarter note E flat | Eb |
| (4) (4) (0) (0) | Enter a half rest | Rh |
| (4) (0) (0) (0) | Enter a whole note G | Gw |