# Motorola Applications Global Network, MAGNET

# Graphical User Programmable Ringer Tone Format

Date: 13  March 2001

Document Reference:

## MAGN GP001 v01 13-03-01Ringer Tone Format

Version: 01

2001 MOTOROLA UK LIMITED. All rights reserved

### Graphical User Programmable Ringer Tone Format

By this feature the user can compose a Ringer Tone of his own, play, save and send the same

to another mobile as SMS. The Maximum number of Notes in a musical tone is 35 notes.

The Music tone, which is sent as an SMS, has the following format:

| Header | Tempo value | Musical Data | Delimiter | Checksum |

Breakup of the Music SMS,

Header: L35&    This is a constant string for all musical tone SMS. [By using  Send As SMS

option user can send composed Ringer Tone to other compatible Motorola phone. To

differentiate Music SMS from Normal SMS, special header is attached.]

Tempo Value: This is either 1 or 2 or 3 or 4 followed by a space(e.g.   2   ). Default is 2.

Musical Data: It consists of  1 or more notes, subject to a maximum of 35 Notes , the format

of which is given below.

Delimiter: &&       This represents the end of musical notes in the tone.

Checksum : Two bytes. The algorithm for calculation of the checksum is given below.

Step1: Calculate one byte  XOR sum of the all the bytes of the musical data starting after

the space in the tempo value(not including space) till the && (not inclusive of &&). Let

us say this is XY.

Step 2: Separate X and Y into two bytes X0 and 0Y.

Step 3: Take 0Y and add hex 30. This becomes 3Y. Call this byte 1.

Step 4: Take X0 and shift right by 4 bits. This becomes 0X.

Step 5: Add hex 30 to 0X. This becomes 3X. Call this byte 2.

Step 6: Two byte checksum is <byte 2><byte1>

More details……

Musical Data:

There are a total of four attributes related to each note in the composed music. Namely,

1. 	Symbol of the note (alphabets from A .. G and a .. g )

2. 	Flatness or Sharpness of the note

3. 	Octave of the note (Lower, Mid, Higher)

- Duration of the note (Value varies from 1 to 6, 1 being the shortest duration)

Out  of  these  four  attributes,  attribute  1  and  4  are  compulsory  and  attribute  2  and  3  are

optional. Once user  enters  in  the  GPRT  feature,  keypad  will  get  converted  into piano  keys.

Keymap will be as follow,

6 -   A  ,  a  	7 -    B  ,  b   	1 -   C  ,  c   	2 -   D  ,  d   	3 -   E  ,  e   	4 -   F  ,  f

5 -   G  ,  g    	0 -   R  ( Rest note ) 	               and  9 -  For toggle between Flat and Sharp(#)

By default the note will be flat until and unless user  makes it as sharp intentionally.

Once user changes this attribute, it will be applicable for all the further notes until he toggles

it again. The capital and small alphabets give visual indication for the flatness or sharpness

of the note respectively.

User  can  see three horizontal  lines corresponding  to each octave. He can select the

desired  octave  by  using  the  side  (volume)  keys.  If  he  doesn t  select  octave,  by  default,  mid

octave will be selected

Press  and  hold  mechanism  is  implemented  for  calculating  the  note  duration.  It  will

give  the effect of real time composition of  the melody. The key press duration is  mapped to

the duration 1 to 6 internally.

All the things, which we have discussed so far, are for real time composition -  on the

handset. Apart from that, user can edit the  composed music. Once the Music Tone Editor is

entered,  user  will  find  a  string  of  Alphabets  and  duration  at  the  bottom.  He  can  move  the

cursor to the desired position and edit the note and its duration as well.

Action list (see footnote) for this will provide following options for the user,

1. 	Play Music

2. 	Save Music

3. 	Erase Music

4. 	Change Tempo

- Send As SMS

Play Music will play the composed music. Save Music will save the same on user  s phone.

Erase  Music  will  not  make  it  blank  but  it  will  overwrite   Motorola  Default  Song   on  user

composed music. First time when user invokes Music Editor, he will see  Motorola Default

Song  Notes on display.

Tempo is the attribute which applies to the whole Tone and not to the single note. By

option  Change Tempo , user can change tempo of the whole melody from 1 to 4.

Here is one user composed note as a sample.

C#+4

Here  C   is note identifier.

#   indicate it is sharp

+   indicates higher Octave and

- indicates duration

As we see  C  and  4  are compulsory attributes and  #  and  +  are optional attributes. So

user can make it C+4 for Flat note (Sharpness not specified), C#4 for Mid octave (Octave not

specified) or simply C4 for flat, mid-octave (Sharpness and Octave both are not specified)

Now let s see one full Tone composed by user,

D#+4C+3D#5F6A6B-3C#2D#-3G-2A4D#+4C+3D#5F6A6B-3C#2D#-3G-

2A4D#+4C+3D#5F6A6B-3C#2D#-3G-2A4D#+4C+3D#5F6A6

If user wants to send it as Music SMS, header and trailer part will attach to it. Let s assume

we have to send the Tone with tempo value 3. The Music SMS will appear as follows,

L35&3 D#+4C+3D#5F6A6B-3C#2D#-3G-2A4D#+4C+3D#5F6A6B-3C#2D#-3

G-2A4D#+4C+3D#5F6A6B-3C#2D#-3G-2A4D#+4C+3D#5F6A6&&<checksum>

At  the receiving end  the  contents between  header  L35&3 and trailer  &&  will be  treated as

Music  SMS.  Its checksum  is  calculated, it  is verified with  that of sent  checksum  along  with

SMS. If it is matching then that is treated as Music SMS.

Note :     There are some constraints while composing the Music tone. Those are,

1 	A# is not available in Lower octave, it will be played as A# mid octave

2 	B# and E# are not available in all octave, they will be played as flat in all octaves

3 	F# and G# are not available in higher octave, they will be played in mid octave

Footnote  *	  :  This  is  the  action  list  for  normal  operation.  If  user  fills  all  rest  notes  another

action list will appear and that contains only one action item that for erasing the Music

END OF DOCUMENT